COMPSYS 723 – Embedded Systems Design

Assignment 1 Report: Low-Cost Frequency Relay (LCFR)

1. Introduction

This report presents the design and implementation of a Low-Cost Frequency Relay (LCFR) using an FPGA-based embedded system running FreeRTOS. The system monitors the frequency of a simulated power network and performs load shedding or reconnection to maintain system stability under abnormal conditions.

The implementation demonstrates key embedded systems concepts including real-time scheduling, interrupt handling, inter-task communication, and hardware/software integration.

2. System Overview

The LCFR system operates as a real-time pipeline:

- Frequency data is obtained from a hardware frequency analyser via an interrupt.

- The frequency is calculated from sampled data.

- The rate of change of frequency (ROCOF) is computed.

- The system determines whether the network is stable or unstable.

- Loads are shed or reconnected accordingly.

- System status is displayed to the user.

3. System Architecture

A system-level diagram illustrating tasks, ISRs, and communication paths is provided below.

[Insert final system diagram here]

3.1 Tasks and Interrupt Service Routines

The system is decomposed into multiple tasks and interrupt service routines to enable concurrent execution.

Interrupt Service Routines (ISRs)

FrequencyRelayISR
Triggered when a new peak is detected by the frequency analyser hardware. It reads the sample count between peaks and sends this value to the FrequencyTask.

KeyboardISR
Triggered by keyboard input. It updates system thresholds for frequency and rate of change of frequency.

PushButtonISR
Triggered by a push button. It toggles the system between normal operation and maintenance mode.

FreeRTOS Tasks

FrequencyTask
Receives the sample count and computes the instantaneous frequency. The result is forwarded to other tasks.

ROCOFTask
Calculates the rate of change of frequency using consecutive frequency measurements.

DecisionTask
Implements the core system logic. It determines whether the system is stable or unstable and generates load control commands.

LoadControlTask
Executes load shedding and reconnection based on decisions. It updates LED outputs accordingly.

SwitchPollTask
Periodically reads slide switch inputs and applies user control logic where allowed.

KeyboardTask
Processes keyboard input and updates threshold values.

VGATask
Displays system information including frequency and system state. Some features remain incomplete and are marked as future work.

4. Task Behaviour

Tasks are classified as either periodic or event-driven.

- Event-driven tasks include FrequencyTask, ROCOFTask, DecisionTask, and LoadControlTask. These are triggered by new data or system events.

- Periodic tasks include VGATask and SwitchPollTask, which execute at fixed intervals.

This separation ensures efficient CPU usage and responsiveness to real-time events.

Task Execution Conditions:

- FrequencyTask executes when new sample data is received from the ISR.

- ROCOFTask executes when a new frequency value is available.

- DecisionTask executes periodically and evaluates system stability based on the latest data.

- LoadControlTask executes periodically and updates outputs based on current system state.

- VGATask executes periodically to refresh the display.

- SwitchPollTask executes periodically to read user input.

This combination of event-driven and periodic execution ensures both responsiveness and consistent system updates.

5. Communication and Synchronisation

Communication between tasks is primarily implemented using FreeRTOS queues.

- ISRs send data to tasks using ISR-safe queue functions.

- Tasks exchange computed values through queues.

Example communication paths include:

- FrequencyRelayISR → FrequencyTask (sample count values)

- FrequencyTask → ROCOFTask (frequency values)

- ROCOFTask → DecisionTask (ROCOF values)

- DecisionTask → LoadControlTask (load commands)

- SwitchPollTask → DecisionTask (user-requested load states)

This structured pipeline ensures that each stage processes data independently while maintaining deterministic system behaviour.

Queues were chosen because they:

- Provide safe communication between ISRs and tasks

- Prevent race conditions

- Maintain data integrity

Shared variables are minimised, and where they are used, access is controlled through structured task interactions.

Queues were selected over shared variables with mutex protection because they provide both communication and synchronisation in a single mechanism. This reduces the likelihood of race conditions and simplifies system design.

Mutexes or critical sections were considered for protecting shared variables; however, they introduce additional complexity and risk of priority inversion. By structuring the system around queues, direct shared memory access is minimised.

6. Design Decisions

Task Decomposition

The system is divided into tasks based on functionality. This improves modularity and allows independent development and testing of components.

Priority Assignment

Higher priority is given to tasks involved in real-time processing:

- FrequencyTask has the highest priority to ensure timely processing of incoming data.

- DecisionTask and LoadControlTask follow to maintain system responsiveness.

- Display-related tasks have lower priority as they are not time-critical.

Use of ISRs

Interrupts are used for handling time-critical events such as frequency measurement. This ensures minimal delay between hardware events and software response.

7. Real-Time Behaviour

7.1 200 ms Requirement

The system ensures that the first load shedding occurs within 200 ms of detecting instability. This is achieved by immediately shedding the lowest-priority active load when the system first enters the load-management state, without waiting for the 500 ms observation interval used for subsequent actions.

7.2 500 ms Load Management

A timer-based mechanism is used to manage load shedding and reconnection:

- If instability persists for 500 ms, the next load is shed.

- If stability persists for 500 ms, loads are reconnected.

This behaviour is implemented using periodic timers and state tracking.

8. FreeRTOS Usage

The system uses several FreeRTOS features:

- Task creation and scheduling

- Inter-task communication using queues

- Interrupt-to-task communication

The use of these features demonstrates correct application of real-time operating system principles.

In addition, several potential multitasking issues were addressed:

- Race conditions were avoided by using queues instead of shared variables.

- Inconsistent state updates were prevented by ensuring only one task is responsible for modifying load control decisions.

- Timing interference between tasks was reduced through appropriate priority assignment.

- ISR-to-task communication was handled using ISR-safe queue functions to prevent corruption of shared data.

9. User Interface

The system uses LEDs and a VGA display to present information.

- Red LEDs indicate load status (on/off).

- Green LEDs indicate loads that have been shed by the relay.

The VGA display shows system information such as frequency and system state.

Note that some display features, including detailed statistics and history tracking, are not fully implemented and are identified as future work.

10. Limitations and Issues

The current implementation has several limitations:

- VGA display functionality is incomplete

- Statistical tracking of recent measurements is not fully implemented

- Some user interface elements are placeholders

These limitations do not affect core system functionality but reduce usability.

An identified design issue is the reliance on periodic task execution for decision and load control, which introduces bounded latency between detection and output. While this meets the timing requirements, a more tightly coupled event-driven approach could reduce latency further.

Additionally, the separation of logic across multiple tasks increases system complexity and requires careful coordination to maintain consistency of shared state.

The assignment requires maintaining a running record of timing measurements, including the five most recent values, minimum, maximum, average, and total runtime.

This functionality is partially implemented; however, full integration with the display system remains as future work.

11. Instructions for Running the System

- Program the FPGA using the provided .sof file.

- Open the project in Nios II Eclipse.

- Build the application.

- Run the program using the JTAG interface.

- Observe system behaviour using LEDs and VGA output.

12. Work Distribution

| Task | Member | Estimated Time (hours) |
| --- | --- | --- |
| Frequency and ROCOF implementation | To be completed | To be completed |
| Load control logic | To be completed | To be completed |
| VGA display | To be completed | To be completed |
| System integration and testing | Both | To be completed |

13. Conclusion

The LCFR implementation demonstrates a complete embedded real-time solution using  both hardware and software components. The use of FreeRTOS enables efficient multitasking and real-time responsiveness. While some interface features remain incomplete, the system successfully meets the core functional and timing requirements.

14. Future Work

- Complete VGA interface

- Implement full statistical tracking (last five values, minimum, maximum, average)

- Improve user interaction features

- Perform further validation of timing constraints